Appendix A

GENERAL AND MECHANICAL OBSERVATIONS Sept. 17 to 30-2012

The purpose of this document is to provide a list of general and mechanical observations that will be used to promote further discussions between the HESCO staff and the Jeff Brown team.

On Monday morning Sept. 17, HESCO personal provided the Jeff Brown team with a presentation of their general overall operation as well as their different production line issues and present solutions which they have been working at to improve the amount of down time. The team then completed a detailed walk around of the facility documenting observations as well as responses to our many questions directed to the HESCO team.

There were some very visible issues that the team noted which would require further investigation as they started a much more in depth inspection on Tuesday morning. One of the major observations which were noted is that there is very little documentation in the way of OPERATOR and PARTS & SERVICE/MAINTENANCE MANUALS available. Without this information readily available to staff, it is very hard to maintain equipment and keep in reliable running order and reducing operating expenses. Service and Maintenance will be addressed in a separate document.  All other observations are documented and divided according to each machine and have been listed in detail: recommendations and or action points are indicated in BOLD.

It should be noted that there may seem to be an excessive number of observations, recommendations or action points required to bring the bottling line up to a standard that would promote high reliability and longevity of the equipment. Not all of these items need to be addressed at once but can be prioritized and incorporated in a schedule that will allow all action points to be addressed over time depending on the success of the business unit.

1-AIR COMPRESSOR:

- The air compressor is designed to provide clean dried air to the blow molding machine which is used to expand the pre-molds after they have been heated to the required temperature. Since this expanding process takes place in just a couple of seconds, it is very critical to have adequate air pressure and volume on demand. As we observed the machine in operation, there were a number of bottles that were not completely blown to the proper shape and size and at the same time it was noticed that the air pressure had dropped a small amount which was enough to affect the process and cause a number of bottle to be rejected.

Upon further investigation of the low air pressure situation, it was discovered that the air supply to the machine was also being used to operate a disinfecting device in the sterile division, the bottle filling machine and the shrink wrapper machine with the shrink wrapping machine being a large consumer of air volume. The air compressor is designed to provide the correct amount of air volume and pressure to operate the blow molding machine only.  During operation it was also noticed that the air compressor was under constant load and that it never had a chance to UNLOAD or catch up to the extra air usage. This can cause over heating and excessive wear on the air compressor internal components that will reduce the service life of the unit.

At this time it is recommended that any extra sterile users, bottling machine and the shrink wrap machine be connected to a different air supply source. There is a second compressor located in the compressor room which supplies the sterile unit that may be capable of supplying some of the extra air requirements. Further calculations would be required to see if the machine can provide any extra capacity.

- During a detailed inspection of the blow molding machine, it was noted that there is considerable rust/dirt in the air filters in the blow molding machine. The rust/dirt appears to be coming from the carbon steel pipes which supply the air from the air compressor to the blow molding machine. Several air samples were then taken at the mold air nozzles and tested for any bacteria. The tests were positive, however it is not known what type of bacteria is present and whether it may be harmful or not. It should be noted that as this air is used to blow the bottles to size, it will carry any dirt/rust/bacteria directly into the bottle. Further research indicated that the manufacture of the blow molding machine as well as the representative from Norland, who was on site during our visit and also manufacture blow molding machines, recommends that Stainless Steel pipe be used to transfer the air from the air compressor to the blow molding machine.

It should be noted that discussions regarding the use of high pressure air hose, indicated that the cost may considerably higher for the hose over the cost of Stainless Steel pipe. When the new pipe is installed, care should be taken to use as few as possible the number of elbows to reduce pressure losses in the piping system.

It is recommended that the air lines between the air compressor/air dryer and the blow molding machine, be changed out to preferably 1 1/4” stainless steel pipe to help reduce the amount of rust and dirt from entering the air supply.

- The air compressor is currently sitting on a cement pad using rags to shim the corners of the base and with no means of anchoring it to the cement pad. Since the air compressor is considered as rotating equipment, it is very important that proper “best practices” be used to anchor the equipment properly to the cement pad.  Excessive vibration on the hard cement surface can cause internal mechanical damage to the compressor crankcase and or internal moving parts reducing service life. Isolation mounts can be sourced in Canada and shipped to CBC - HESCO via DHL.

It is highly recommended that the compressor be re-installed with the use of VIBRATION ISOLATERS between the compressor frame and the cement pad/floor, and leveled properly.

- The current air cleaner elements inside the air cleaner housings are the wrong size and as well they appear to have been over tightened, causing damage to the filter paper pleats. This reduces the filter efficiency that reduces the ability to effectively filter the air of small dust particles. Excessive dirt will cause excessive wear on cylinders/pistons.

It is suggested that a possible larger remote mount style air cleaner be installed that will draw air from outside of the current location or some type of piping system which draws air from outside the current location to the compressor filter housings.

- Maintenance personal indicated that there have been a number of issues with the compressor relating to the type and weight of oil used.  Some damage did occur to the connecting rod bearings in the crankcase and were changed out with used ones from the old failed compressor. They have since sourced synthetic SAE 140 oil which appears to be working well with the used bearings but consideration should be taken to purchase a set of new connecting rod bearings before further damage occurs to the crankshaft journals from the old original bearings.

There are also indications that the 3rd stage cylinder UNLOADER valve is leaking, most likely due to high heat from continual operation under load. Maintenance indicated that the valves are in good shape but gaskets and seals are needed for further preventative maintenance.

A new set of 4 connecting rod bearings, complete gasket kit including 4 sets of un-loader valve seal sets should be ordered as soon as possible.

2-AIR PURIFIER:

- The air purifier is designed to remove any remaining oil residue and water/moisture from the air before it flows to the blow molding machine. As noted in the air compressor section above, the current unit is operating at the maximum allowable capacity due to the extra requirements of the air supply for the bottling and shrink wrap machines.

As previously noted it is recommended that the bottling machine and the shrink wrap machine be connected to a different air supply source.

- The cooling coil inlet (class F) filters and outlet (Class T) filter elements have to be replaced when differential pressure reaches 0.07 Mpa. Unfortunately the air purifier unit is not equipped with pressure gauges to indicate the actual differential pressure across the filters. Since this procedure is very important so that the filters can be effective in removing any oil residue and or moisture from the air, it is recommended that they be replaced on a regular schedule (example 6 month intervals). It should also be noted that these filters are NOT TO BE WASHED but replaced with new ones and care must be taken to not touch the filter material with your hands

It is recommended that a minimum of 2 sets of new filter elements be ordered and kept in inventory so that the elements can be changed when required.

- Mounted on the bottom of the Inlet class F filter housing, the outlet class T filter housing and the air exchanger unit drain are auto drain valves. These are designed to open at preset intervals to allow any water or oil residue to drain from the filter units.  One of the valves is missing a solenoid coil and so is not working to allow any water/oil residue to drain from the filter housing.

Maintenance indicated that spare solenoids are on order but there should also have a controller as spare parts.  Please refer to operator’s manual page 10-24 for the correct settings for the INTERVAL and DURATION of the solenoid drain valves to operate.

- Inside of the air purifier unit is a cooling core which needs to be serviced regular. With the high humidity of the climate and the current location of the unit in a dusty dirty area the cooling fins can become clogged with dirt and moisture. This clogging can reduce the amount of heat which can be exchanged affecting the ability to purify the air.            Maintenance indicated that they are maintaining the cleanliness of the cooling core on regular bases.

- The air purifier is equipped with a cooling fan which has been replaced with an alternate unit.

It is recommended that a new fan from the manufacturer be ordered and installed in order to achieve proper operation and cooling effectiveness.

3-BLOW MOLDING MACHINE:

- The blow molding machine is currently operating quite well with most of operation problems caused from low air pressure which has been addressed in previous statements. Mechanically the unit is in good running condition with only a few minor air leaks and oil leaks. There is however several important issues that need to be addressed before major problems and loss of production occurs.

- The main drive chain which moves the preform’s through the heater tunnels, molds and to the bottle loading and unloading section is showing signs of excessive wear. The adjustment device indicates that the chain has lengthened a substantial amount due to incremental wear in each chain link.

It is recommended that the chain be replaced before excessive wear takes place on the drive and idler chain sprockets. When these drive chains are replaced they should be lubricated with FOOD GRADE GREASE LUBRICANT.

- The secondary drive chain is a smaller chain which rotates the pre-molds for even heat distribution as they travel around the machine. This chain has reached it maximum adjustment which indicates that he chain is wore out due to incremental wear in each chain link.

It is recommended that the complete chain be replaced before excessive wear occurs on the drive and idler sprockets. When these drive chains are replaced they should be lubricated with FOOD GRADE GREASE LUBRICANT.

- There are 2 electric drive motors which power the main and secondary chains. The main chain drive motor was replaced about 1 year ago from local inventory. A backup motor has NOT been ordered and as well there is NO secondary drive motor in inventory.

It is recommended that 1 each of the drive motors be ordered for inventory. In addition it is recommended that the preform conveyor drive motor be ordered as well.

- When the new bottles are unloaded from the machine they are placed on a conveyor to the bottle filling machine. The proximity switch which controls the conveyor movement is faulty preventing the conveyor from operating properly and safely. This is a simple fix once the proximity is sourced.

It is recommended that the proper switch be ordered and installed so that machine will operate in a proper and safe mode.

4-BOTTLE FILLING MACHINE:

- The bottling machine is plagued with a number of mechanical and electrical issues. The most important item is the bottle cap installer assembly. The cap spinner includes magnetic drives which disconnect or stop spinning the cap when the cap is tight, 2 of the 3 cap spinner assemblies are not operating properly. The spinner assemblies were taken apart and found to be very dirty. All parts were cleaned and assembled with new seals. The bearings are NOT in good shape and should be changed in the next 3 months. It is suggested that 2 new set of 6 bearings and 3 seal kits be ordered, 1 to be installed 1 for stock and future repairs.

- Part of the cap installing unit includes a 3 position wheel which holds the bottles from turning during the capping process. Mounted on the position wheel are 3 small plates which support 3 small pins each. These small pins hold the bottle from turning while the cap is installed. With 2 of the cap spinner assemblies not releasing properly after the cap is installed the spinning bottles have damaged the pins. Maintenance personal indicated that they can source the pins locally. The manufacturer indicated that there is a new style plate insert available which was ordered but turned out to be the wrong one. These should be returned for credit and the proper plates reordered.

It is recommended that this be addressed immediately so that all bottles have the cap installed and torqued properly.

- The cap conveyor is presently operated in manual mode as the micro controller and the motor drive VFD are missing from the machine. Maintenance indicated that the controller is in stock but the motor drive has not been ordered as of yet.

These VFD should be ordered and installed so that the machine will run properly and will reduce the number of personal required to operate the machine.

- The cap conveyer includes a UV light which disinfects the caps just before they are placed on the bottles. Maintenance personal indicated that this unit requires service. One issue is that the caps keep getting stuck at the top of the track and an operator has to put his finger into the track to free the caps loose.

It is recommended that the correct UV Bulb assembly be sourced and specifications checked for operation life. The bracket assembly which supports the UV assembly requires some alignment for correct operation. Again this is an easy fix without spending a lot of money.

- UV (ultra violet) unit is currently out of order due to no spare parts in inventory. The Quartz Glass Tube which houses the UV Bulb is broken. This is due to the deterioration of the rubber seal which seals the water and the tube. The Rubber seal is old, cracked and is very brittle, excessive force was used to compress the rubber seal to establish as good seal which was too much and broke the tube. Also related to the glass tube is the internal wiper assembly. The qty. 4 wiper seals are old, distorted and do not fit properly causing the wiper to not function. The Jeff Brown team is currently trying to source UV tubes and seals in Canada.

It is recommended that qty. 4 new Quartz Glass Tubes, 2 UV bulbs, 6 new tube to end cap seals, 8 new wiper seals, 1 wiper actuation shaft and seals be ordered and kept in inventory to minimize down time.

- On the primary supply pump skid there are qty. 2 water filters. These filters need to be changed on a regular base to maintain the proper water quality. Currently there are no spare filter elements in inventory. Since the filtration/UV unit is critical to proper operation of the filling machine it is very important to keep these items in stock.

The smaller filter canister is a PALL unit and parts have been sourced in Canada. It appears that the wrong filter element is being used in the small filter unit as an EPOXY has been used to hold the filter in position. Research has indicated that the proper elements TWIST in place which should be considered.

The larger filter canister has no numbers available and so sourcing parts are dependent on the bottle filling machine manufacturer. This filter was removed due to excessive pressure loss.

A new canister, weld fittings, clamps, seals and filter elements can be sourced in Canada and shipped via DHL that could be installed into the piping after the UV unit to filter out any dead microbial from the UV unit.

The Jeff Brown team is in contact with the manufacturer of the unit in India regarding spares as well as PALL FILTERS in Canada.

- The conveyor which moves the bottles from the bottling machine through to the shrink wrapper is powered by an electric motor and gear box assembly at the shrink wrapper end. During our inspection it was noted that gear oil was leaking from the gearbox onto the floor and making a mess in general. Several bolts were found to be loose, were retightened and any leaking oil wiped up. After a few hour of operation it was noted that the oil was still leaking from the drive assembly but at a reduced rate. Upon further investigating it became apparent that the oil is leaking internally from the shaft seal between the gearbox and motor. The excess oil was wiped from the bottom of the motor and was continued to be monitored. It was confirmed that the internal seal is leaking. This seal needs to be replaced as soon as possible. If left in the current state, the oil will continue to leak internally into the electric motor which will eventually cause an electrical short damaging the motor.

It is recommended that the bottling machine manufacturer be contacted for a new seal.

5-SHRINK WRAP MACHINE:

The shrink wrap machine is perhaps the most troublesome unit on the HESCO water bottling line. It has many worn parts and is in need of a complete major refurbishment but due to the simplicity of the unit it is well worth spending the time and money to refurbish the unit.

- The machine operates at 200 C, so a number of heat related issues need to be addressed with the most important being the circulation fan in the heat tunnel. The sides of the tunnel are lined with electric heating elements and the fan forces air over these elements towards the shrink wrap material. The original 3000 RPM fan motor failed and was replaced with a 1500 RPM motor and a homemade fan assembly. The combination of the slower speed motor and the smaller less efficient fan, which cannot circulate the hot air properly, causes hot/cold spots, burns the electrical wiring and in general creates a poor quality shrink wrap job. Many of the 6 packs of bottles are not shrink wrapped properly causing bottles to fall from the pack and make them hard to handle.

- A potential problem that can affect the tunnel temperature is the curtains located at each end of the tunnel. The curtains help reduce the amount of heat exiting the tunnel as the bottle packages move in and out of the tunnel that in turn reduces the temperature inside the tunnel. The current ones are in very bad shape and require replacement as soon as possible.

- The sealing knife is moved upwards by 2 air cylinders during the sealing process. One of the air cylinders has been damaged and repaired as much as possible but not properly and will require replacing the cylinder.

- Located on the sealing knife is a temperature sensor. It is currently not mounted correctly and is causing an improper temperature which could affect the plastic cutting process.

- Heating elements in the tunnel have wiring failures about every 2-3 weeks. It was discovered during our visit that one of the main power wires was shorted due to a piece of tubing being used for a fuse. This causes other wires to overload and short out on a regular base.  The high heat insulation also is showing signs of deterioration. A higher operating temperature wire will be sourced but in the meantime a heat shield to help protect  the wiring from the radiant heat was designed and installed to help prolong the life of the wiring. In the meantime the cause of the heating elements wiring failure was discovered will be repaired.

- The machine has 2 locations where the rolls of shrink wrap plastic are positioned on rollers. The plastic sheeting is then routed through a serious of rollers to where the product is wrapped. It was found that the rollers have guides which keep the plastic centered were not located properly. This caused misalignment of the 2 plastic sheets where they come together which then causes a poor quality joint and a poor quality package wrap.

It was recommended to maintenance personal to realign all of the rollers and guide wheels so that the plastic sheets will stay in alignment and produce a better package.

After the entire machine was inspected, a list of parts requirements was sent to the manufacturer who has responded with the following list noting that all parts in stock. It should also be noted that if the following parts are purchased and installed the shrink wrapping machine will provide many years of trouble free operation.

LIST OF RECOMMENDED SPARES FOR SEMI-AUTOMATIC

SHRINK WRAPPING MACHINE

Item                                           			Qty. @ US ea.                 Total USD

- Sealing Rubber 				2 @ 45.00 ea		90.00

- Hot Air Fan Motor 				1 			270.00

- Bottom Sealing Arm Pneumatic Cylinders 	1               		225.00

- Heater Wire 					100 mtr. @ 2.70 mtr. 	270.00

- Proxy Switch 					2 @ 27.00		54.00

- 3’’ Wide Sealing Tape Roll 			2 @ 54.00		108.00

- Solenoid Valve + Coil 				1 			144.00

- Tunnel Temp. Controller + Sensor 		1 			63.00

- Sealer Temp. Controller + Sensor 		1  			63.00

- Tunnel Heaters 				10  @ 18.00		180.00

- Sealer Heaters 				2 @ 63.00		126.00

- Tunnel Curtain Cloth 				2 @ 27.00		54.00

- Pneumatic Cylinder Kit 			2 @ 27.00		54.00

- Cooling Fans     				2 @ 108.00		216.00

TOTAL COST 					$1917.00

6-ELECTRICAL DISTRIBUTION SYSTEM:

HESCO Water

Electrical Observations

Jeff Brown team members reviewed the entire electrical distribution network within the CBC HESCO compound. Several issues were documented and some should be addressed immediately.

- The main disconnect at the main power pole is not operating correctly. The switch mechanism on one of the phases is not making proper contact. This can lead to one phase running at a lower amperage load limiting the entire distribution system.

The power provider should be made aware of this situation as soon as possible.  The utility company may have come out since the issue was discovered, as reviewing the equipment again seems to indicate that the switch has been serviced.  Need to check with the maintenance engineer and the utility on this.  The switch looks to be old and worn out and should be replaced as there are portions of it out of alignment and in need of replacement rather than adjustment.

- Phase balancing in the main distribution panel is very unbalanced due to lack of procedures for installing new electrical loads throughout the compound. There are number of areas where additional single phase loads have been added without determining which phases have capacity in comparison to other phases.  This may be due to the lack of availability of the appropriate phases in the vicinity of the additional loads.  As well, extra electrical loads have been tapped off of the outputs of existing breakers designed for specific loads. This is a dangerous practice as the additional loads are not protected properly if shorts or grounds take place as the breaker size they are sub fed from is too large to protect the smaller conductors and loads. It can cause fires, injury to personal and major electrical equipment damage. It is a significant undertaking to address this issue but there should be some plan put into place going forward to employ proper practices for adding electrical loads to the distribution system. A secondary plan could be adopted to repair the current system over the course of several months by adding terminal block arrangements and additional breakers.

- Stand-by generator switch gear in its current design is very dangerous in that there is the possibility to connect the generator and the power provider at the same time which can have disastrous effects to equipment and personal. Switchgear can be installed in the power distribution room that would provide an interlock between the generator and the power provider so that it is impossible for the 2 power sources to be connected at the same time.

HESCO Water

Electrical Maintenance Recommendations

- All cables in below grade channels should be secured or suspended within 50 mm of the bottom of the grating or cover to avoid the cable being submerged in water at times other than in extreme cases.

- If possible, from this point forward, when adding new loads to the system, best efforts should be made to adding those loads to underbalanced phases of the electrical system.

- All electrical equipment such as contactors, relays, breakers, motor starters etc., in the future, should be mounted to the wall or ceiling on some sort of a non-conductive material and firmly mounted to that surface.

- As much as possible, electrical equipment should be in some sort of enclosure.

- In the future, wiring tap-offs from breakers or at junction points, should always have a circuit breaker protecting those tap offs and their wire size and be located between the source and the tap-off.

- All electrical cabling should be adequately supported if installed on the walls or ceiling and if run along the floor protected from being stepped on.

- All cable splices should be done with wire nuts of ring terminals and bolts and insulated and protected from the elements with vinyl tape if indoors and rubber and vinyl tape if outdoors.  Screw terminal blocks should be used indoors only and mounted to a wall of frame.

- Breakers should be sized and sourced for the Sterile Compressor as well as the cooler and chiller/filter skid.  A terminal block should be used and mounted at the feed wiring for the Blow Mold Compressor and then tapped off from that terminal block for the breakers for the Blow Mold Compressor, The Sterile Compressor, the Cooler and the Chiller/Filter Skid.  From there the equipment can be safely connected.

- The electrical room should be locked at all times.

- The utility closet where Sonel’s metering equipment is located should be locked at all times.

- Sonel needs to be pressured into replacing the main, pole mounted, disconnect switch with either a new one or a refurbished unit as the existing unit has been damaged as well as is in need of adjustment.  In short, it is worn out.

HESCO Water

Electrical Spare Parts

A stock of spare parts for all electrical equipment installed at HESCO should be developed and purchased in order to avoid unnecessary downtime caused by waiting for electrical parts to be purchased and delivered to the plant.  Typically these items are not expensive and the cost and inconvenience of the associated downtime is far greater.

| HESCO Water Spare Parts List |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Spare Part Item | Use | Description | Qty Installed | Qty Spares | Part Number Description |
| Circuit Breaker | Main Dist | 160 Amp Feeder Breaker | 2 | 1 | Merlin Gerin Compact NS 160N |
| Circuit Breaker | Main Dist | 100 Amp Feeder Breaker | 1 | 1 | Merlin Gerin Compact C 125N |
| Circuit Breaker | Main Dist/Pres Pump | 250 Amp Feeder Breaker | 2 | 1 | Merlin Gerin NS 250 N |
| Circuit Breaker | Main Dist | 125 Amp Feeder Breaker | 1 |  | Merlin Gerin Compact R 125 |
| Circuit Breaker | Main Dist | 25 Amp Feeder Breaker | 1 |  | Merlin Gerin C60N/C25 |
| Circuit Breaker | Main Dist | 50 Amp Feeder Breaker | 3 | 1 | Merlin Gerin C 60N/C50 |
| Circuit Breaker | Main Dist | 16 Amp Feeder Breaker | 1 |  | Merlin Gerin 16A |
| Circuit Breaker | Main Dist | 32 Amp Feeder Breaker | 1 |  | Legrand C 45N/C32 |
| Circuit Breaker | Main Dist | 40 Amp Feeder Breaker | 1 |  | Legrand C 40 |
| Circuit Breaker | Main Dist | 10 Amp Feeder Breaker | 1 | 1 | Legrand C 16 |
| Circuit Breaker | Main Dist | 32 Amp Feeder Breaker | 1 | 1 | Merlin Gerin DT 40/C32 |
| Circuit Breaker | Main Dist | 20 Amp Feeder Breaker | 1 |  | Merlin Gerin C 32N/C20A |
| Circuit Breaker | Main Dist | 20 Amp Feeder Breaker | 1 | 1 | Legrand C 20 |
| Circuit Breaker | Shrink Tunnel | 32 Amp Branch Breaker | 1 |  | Merlin Gerin C 32N |
| Circuit Breaker | Shrink Tunnel | 60 Amp Branch Breaker | 1 |  | Merlin Gerin C 60N/C63 |
| Circuit Breaker | Shrink Tunnel | 25 Amp Branch Breaker | 1 |  | Legrand C 45N/C25 |
| Circuit Breaker | Shrink Tunnel | 16 Amp Branch Breaker | 1 |  | Legrand C 16 |
| Circuit Breaker | Shrink Tunnel | 10 Amp Branch Breaker | 1 | 1 | Legrand C10 |
| Circuit Breaker | Shrink Tunnel | 40 Amp Branch Breaker | 1 |  | Merlin Gerin DT 40 |
| Circuit Breaker | Shrink Tunnel | 63 Amp Branch Breaker | 1 |  | Legrand C 63 |
| Circuit Breaker | Label Shrink | 4 Amp Branch Breaker | 1 |  | Merlin Gerin 60HB1G 3 Pole |
| Circuit Breaker | Label Shrink | 16 Amp Branch Breaker | 1 |  | Merlin Gerin 60H C4 |
| Circuit Breaker | Filling Machine | 6 Amp Branch Breaker | 4 | 2 | Merlin Gerin C 60H C6 |
| Circuit Breaker | Filling Machine | 10 Amp Branch Breaker | 1 |  | CHNT DZ47-60-C10 2 Pole |
| Circuit Breaker | Filling Machine | 5 Amp Branch Breaker | 1 |  | CHNT DZ47-60-C5 3 Pole |
| Circuit Breaker | Filling Machine | 5 Amp Branch Breaker | 3 | 1 | CHNT DZ47-60-C5 2 Pole |
| Circuit Breaker | Blow Mold Machine | 60 Amp Branch Breaker | 1 |  | CHNT DZ47-60-C60 |
| Circuit Breaker | Compressors | 100 Amp Branch Breaker | 1 |  | Merlin Gerin NS 100 N |
| Circuit Breaker | Bore Hole Pump | 32 Amp Branch Breaker | 1 |  | Telemecanique C 60N C32 |
| Circuit Breaker | Pressure Pumps | 25 Amp Branch Breaker | 1 |  | Telemecanique C 60L C25 |
| Contactors | Filling Machine | 2.5 to 4 amp contactor | 2 | 1 | Telemecanique GV2MEO8 2.5-4 Amp |
| Contactors | Filling Machine | 1.6 to 2 Amp Contactor | 1 |  | Telemecanique GV2MEO7 1.6-2 Amp |
| Contactors | Compressors | 63 Amp Contactor | 3 | 1 | CHNT CJX1-63 |
| Contactors | Compressors | Contactor | 1 |  | Telemecanique LC1C 4011 |
| Contactors | Bore Hole Pump | Contactor | 1 |  | Telemecanique LC1 DGS11 |
| Contactors | Pressure Pumps | Contactor | 1 |  | Telemecanique LC1 DO910 |
| Contactors | Pressure Pumps | Contactor | 1 |  | Schnieder LC1 012 |
| Relays | Shrink Tunnel | Relay | 1 | 1 | Telemecanique LC1K1610M7 |
| Relays | Shrink Tunnel | Relay | 1 |  | Telemecanique LC1K1210M7 |
| Relays | Shrink Tunnel | Relay | 2 | 1 | Telemecanique LC1K0610M7 |
| Relays | Shrink Tunnel | Relay | 1 |  | Telemecanique LP1K0610BD |
| Relays | Filling Machine | Relay | 2 | 1 | Telemecanique LR2K0307 |
| Relays | Filling Machine | Relay | 4 | 2 | Telemecanique LC1K0910M7 |
| Relays | Filling Machine | Relay | 1 | 1 | PLA Relay 13C/OMP3 240-S |
| Relays | Bore Hole Pump | Relay | 1 |  | Telemecanique LC1 D25 |
| Auxillary Contacts | Shrink Tunnel | Relay | 1 |  | Telemecanique LA1KM11 |
| Relays | Blow Mold Machine | Relay | 4 | 2 | Telemecanique CJX20910 |
| Relays | Blow Mold Machine | Relay | 1 |  | Telemecanique LC1AO910 |
| Relays | Blow Mold Machine | Relay | 1 | 1 | Omron LY2NJ |
| Relays | Blow Mold Machine | Relay | 1 | 1 | Matsushita HC4-H-AC 240V |
| Relays | Compressors | Relay | 1 |  | CHNT JZC4-22 |
| Relays | Compressors | Relay | 1 | 1 | Delixi JS14P |
| Overload Relays | Bore Hole Pump | Overload Relay | 1 |  | Schnieder JIS C8201-4-1-5-1 |
| Overload Relays | Pressure Pumps | Overload Relay | 1 |  | Telemecanique LRD14 |
| Overload Relays | Pressure Pumps | Overload Relay | 1 |  | Schnieder LR2D13 |
| Prox Sensor | Blow Mold Machine | Proximity Sensor | 1 | 1 | Telemecanique XSA-AO5020 |
| Photo Sensor | Filling Machine | Photo Sensor | 1 | 1 | Balluff BOS 18K-UU-IPC F4C |
| Pressure Switch | Pressure Pumps | Pressure Switch | 1 | 1 | Telemecanique MGB008 |
| Terminal Blocks | General |  | Unknown | 20 | Wieland 10 Pole Euro Style |
| Din Rail | General |  | Unknown | 6 Meters | Any Manufacturer |
| Ring Terminals | General |  | Unknown | 50 each | Any Brand of 2, 4 and 6mm Wire |
| Fuses | Blow Mold Machine |  | Unknown | 4 | RT18RT14RO1640A 500V GB13539-1-92 |
| Electrical Tape Vinyl | General |  | Unknown | 12 Rolls | Any Manufacturer |
| Electrical Tape Rubber | General |  | Unknown | 4 Rolls | Any Manufacturer |
| Cable Straps | General |  | Unknown | 100 | Any Manufacturer |
| Nylon Tie Straps | General |  | Unknown | 500 | Any Manufacturer |

From the above list Fai Emmanuel chose 12 items that he felt were most critical.  The cost of those 12 items is approximately $1,300.00 USD.

7-MAIN WATER SUPPLY PUMPS:

The most important pump of course is the well bore pump. The current pump is new about 1 year ago and should operate trouble free for 3-5 years but since this pump is so vital to health services and the hospital, it is strongly recommended that a spare pump be kept in stock. Things like electrical storms and unreliable electrical service, which seems to be a normal happening, can damage a pump very easily.

Also located next to the main water reservoir close to the well are 2 GRUNDFOSS water pumps which supply water to the HESCO compound, sterile division and to an overhead reservoir which supplies water to the main hospital approx. 1.6 kilometers down the hill from health services. The pumps are set up to operate on a pressure switch in conjunction with a pressure tank. The pump builds pressure in the storage pressure tank until a pressure switch actuates and stops the pump at a preset pressure.  As water is used, the pressure in the tank decreases to appoint where the pressure switch turns the pump on recharging the tank.  The pumps were designed and set up in parallel so that one pump is always on standby.

However the current 2 pumps are set up to operate in manual mode only as the pressure tank is out of commission with a leaky bladder. The unit is too old to buy parts for so a new pressure tank sized correctly for the application should be considered. One of the 2 pumps operates during the day and is shut down at night.

It was also noted that the pumps are capable of producing pressure too high for the Guest House and the Office sinks and toilet control valves but the high pressure is required to operate the new Distiller unit which was commissioned while the Team was at HESCO. A pressure reducing valve will be required to lower the pressure to all facilities except the Water Distiller unit. Discussions also indicated that the third pump will be installed to supply the distiller unit but this could prove to a more costly approach since a new pipeline will have to run from the pumps to the sterile building.

There are also 2 extra pumps sitting next to the current operating pumps. These 2 pumps have been disassembled and accessed for repair. There are enough parts to build 1 out of the 2 pumps with the addition of a new gasket/seal kit and new mechanical seal. Maintenance personal were briefed on the theory of operation, maintenance and repair of the pumps. With a 3rd pump in a “ready to use” mode, there would always be one pump on hand for installation as a standby.

NOTE: It should be taken very serious that the well pump and these 2 system pumps mentioned above supply water for the entire CHC Health Unit as well as the CBC Hospital and that spare pumps should be considered mandatory. It is also highly recommended to purchase a new pressure tank so that the supply system can operate in a very efficient and effective manner.

8-STAND-BY POWER GENERATOR:

The current stand-by generator is a very high quality unit and is in very good shape. The Mercedes engine is sized larger than is required and operates at a slower speed. The engine was started and put under load for the Jeff Brown team to evaluate the general operation of the engine, governor, generator and control panel.

The only major issue that needs to be addressed as soon as possible is that the diesel fuel should come from a proper fuel tank and NOT from an open barrel as it currently is operating from. Barrels have a tendency to collect water and water in a diesel engine fuel system will cause considerable damage as well as result in down time when the unit is required.  It is recommended that this issue be addressed immediately.

Minor issues include replacing the phase volt meters so that accurate voltages of each phase are available to the operator.

A Maintenance schedule should be set up for servicing of the engine and generator. It is common for a stand by generator to be FORGOTTEN since it does not operate on a steady base but it requires oil changes and servicing as if it was a major piece of equipment running in a normal operation.

9-CONCLUSIONS:

The Jeff Brown team spent a considerable amount of time inspecting the machines and promoting discussions with the mechanical and electrical maintenance personal as well as the machine operators about the general operation of the machines, recognizing potential trouble spots both past and present, maintenance procedures, spare parts inventory and general overall felling of the machines.

These machines were purchased and installed in 2005/2006 and have experience a number of maintenance issues during this period which has affected production and sales to a very high degree. The machines were not the highest quality on the market to begin with but if serviced and maintained on a strict maintenance schedule along with a good selection of spare parts, these machines will operate with a much higher reliability and fewer shutdowns for many years to come.

A formal maintenance schedule should be created in conjunction with operations, maintenance and machine operators personal. At completion and acceptance of the document, it should be implemented and followed very close.

To support an effective maintenance program also requires a good spare parts program. A list of spare parts should be created based on past history as well as potential future needs of each machine that were noted during discussions.

CBC HESCO has some very knowledgeable mechanical and operations personal on staff and should be proud of these people. They have been doing an extremely good job at maintaining and servicing the machines as well as minor and major repairs on them. But at the same time they have been very limited as to what can be done and what should be done due to lack of spare parts. As noted prior, if a good maintenance program was implemented including a proper spares inventory these machines will operate with less downtime for many years to come. The Jeff Brown team would be willing to help create an effective maintenance and spare parts program.